IATG SAFERGUARD TRAINING MODULES

MODULE FO04 – FIELD AND TEMPORARY STORAGE QUANTITY DISTANCES

(IATG 04:10)

MINI-EXERCISE

INSTRUCTOR’S SOLUTION

SCENARIO

You are deployed on a UN operation in a foreign country.  You currently work in a logistics role.  You are tasked to run a Field Storage Area (FSA).  A calculation of the total stockpile revealed an obvious requirement for 5 individual Field Storage Sites (FSS).

There is one area that is naturally traversed due to the slope of a small hill which creates a natural horseshoe shape around one large area of flat ground.  This could be considered as a traversed FSS if it is used.

No other topographical features exist that would be suitable as a natural traverse.

There is a farm in the vicinity, which should be considered as a site that is used by the general public.

DATA

The 5 FSS that you have identified are differentiated by the Hazard Division of the stock held within each FSS.  The FSS are as follows:

| FSS | Hazard Division of stock |
| --- | --- |
| 1 | 1.1 |
| 2 | 1.1 Vital Stocks |
| 3 | 1.2 |
| 4 | 1.3 Propellant |
| 5 | 1.4 |

In all cases, the NEQ is less than 50% (NEQ<50%) of the gross weight.

In all cases, the FSS comprise 20 x Field Storage Modules (FSM).

The FSS are linear in layout, as the individual Field Stack Modules within each FSS are either side of a road.

You decide to use the traversed site as FSS 1 (Hazard Division 1.1 stock).

QUESTIONS

1.	What is likely to be the minimum length of road that each FSS will take up (based on 20 x FSM, spread out along both sides of a road)?

50m minimum

2.	You also have to locate an area for empty packaging.  How far should this be from each of the FSS containing ammunition?

100m

3.	There is quantity of ammunition that is awaiting disposal by demolition because it is ‘shelf life expired’.  This will go into another Field Storage Site.  How far should this be from other FSS containing ammunition?

300m

4.	What is the minimum separation distance between the FSS?  Complete the table to show distances in meters.  If a permutation gives two different readings when considered using the table from IATG 04:10, use the larger distance.   Include the distances for the packaging site and the ‘shelf life expired’ ammunition.  The stock for disposal is HD 1.4.

| FSS | 1 (traversed) | 2 | 3 | 4 | 5 | Packing | For disposal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (traversed) | N/A | 200 | 100 | 100 | 100 | 100 | 300 |
| 2 | 300 | N/A | 100 | 100 | 100 | 100 | 300 |
| 3 | 100 | 100 | N/A | 100 | 100 | 100 | 300 |
| 4 | 200 | 100 | 100 | N/A | 100 | 100 | 300 |
| 5 | 100 | 100 | 100 | 50 use 100 | N/A | 100 | 300 |
| Packing | 100 | 100 | 100 | 100 | 100 | N/A | 100 |
| For disposal | 300 | 300 | 300 | 300 | 300 | 100 | N/A |

5.	Given that the Field Storage Area contains ammunition of Hazard Division 1.1, what is the minimum distance from the closest point of the FSA to the farm, which is an Inhabited Building?

500m

6.	Using the sketch map of the general area to locate the Field Storage Area, mark on a solution showing the various Field Storage Sites (including for disposal ammunition and the packaging).  Remember to apply the minimum distances from Question 4 to the individual Sites, and also the edge of the complete Field Storage Area to the Inhabited Building.

1

Road

Traverse area

1

Pack

5

2

3

4

Disp

500m OQD from FSA to IB

Each square is 100 meters.

(Instructor:  there will be many permutations, particularly given the distances involved.  The intent is to get the students thinking about location of sites relative to each other.  If they struggle to get started, tell them to apply some distances from the traversed are (FSS 1), perhaps to FSS 5 as this is a low distance (100m, as it is HD 1.4).  Suggest they then place other FSS to meet the necessary distances from FSS 1 ad FSS5, and continue from there.

In their solutions, the FSS should be along the road itself to represent the lines of FSM either side of a track.  The exceptions are FSS 1 in the traversed area, and perhaps the packaging area. Also discuss the merit of having to drive through the stockpile of ‘shelf life expired ammunition’ – would this be better being off the main circuit through the area?).